Exam Objective:

Design and configure a network where two separate LANs, each containing a switch and two PCs, are connected via two routers. Implement static routing to enable full connectivity between all devices across the two LANs.

IP Addressing Scheme:

- LAN 1 (Connected to Router1):

  - Network: 192.168.10.0/24

  - Router1 (LAN Interface): 192.168.10.1

  - PC1: 192.168.10.2

  - PC2: 192.168.10.3

- LAN 2 (Connected to Router2):

  - Network: 192.168.20.0/24

  - Router2 (LAN Interface): 192.168.20.1

  - PC3: 192.168.20.2

  - PC4: 192.168.20.3

- Router Interconnection:

  - Network: 10.0.0.0/30

  - Router1 (Serial Interface): 10.0.0.1

  - Router2 (Serial Interface): 10.0.0.2

Tasks:

- Device Configuration:

  - Assign appropriate device names to all routers and switches.

  - Configure IP addresses on all router interfaces as per the IP addressing scheme.

  - Ensure all interfaces are activated.

- PC Configuration:

  - Assign IP addresses and subnet masks to all PCs.

  - Set the default gateway on each PC to point to the LAN interface of the connected router.

- Static Routing:

  - On Router1, configure a static route to the 192.168.20.0/24 network via the 10.0.0.2 next-hop address.

  - On Router2, configure a static route to the 192.168.10.0/24 network via the 10.0.0.1 next-hop address.

- Connectivity Testing:

  - From PC1, ping PC3 to verify connectivity.

  - From PC2, ping PC4 to verify connectivity.

  - If pings are unsuccessful, utilize troubleshooting commands such as ping and tracert to identify and resolve issues.

Submission Requirements:

- A Packet Tracer file (.pkt) with the completed network configuration.

- A brief report detailing the configuration steps taken, any challenges encountered, and how they were resolved.

Assessment Criteria:

- Correctness of device configurations and IP addressing.

- Proper implementation of static routing.

- Successful end-to-end connectivity between PCs in different LANs.

- Clarity and thoroughness of the accompanying report.

Additional Resources:

- For guidance on configuring static routes, refer to Cisco's official documentation or relevant networking textbooks.

- Ensure to save your Packet Tracer project frequently to avoid loss of work.