KITCHEN PLANING AND ORGANISATION

The purpose of kitchen planning and organization is to produce the right quantity of food of the highest standard for the required number of people, on time by the most effective use of staff, equipment and materials. Regardless of whether the organization is simple or complex, the factors which have the greatest effect on the organization will be the menu and the system used to prepare and present the menu items. Therefore, the kitchen organization will vary mainly due to the size and the type of establishment ie in schools, hospitals, fast food kitchens, open kitchen etc.

KITCHEN DESIGNING

Kitchen designing focuses on the physical and technical layout of the kitchen space including:

- Structure

- Equipment placement

- Lighting

- Ventilation

- Utilities

- Finishing materials

## Major Components of Kitchen Design

### 1. Structural Design

Includes:

- Floor materials (non-slip, easy to clean)

- Wall finishes (washable, heat resistant)

- Ceiling height (for ventilation and heat control)

- Door width (for trolley movement)

### 2. Equipment Layout

Equipment should be placed according to workflow:

- Cold storage near receiving

- Preparation tables near storage

- Cooking equipment near preparation

- Service counters near cooking

Large hotel kitchens such as those in Serena Hotel follow zone-style layouts with separate sections for pastry, garde manger, and hot kitchen to increase specialization and efficiency.

### 3. Ventilation System

- Exhaust hoods above cooking ranges

- Proper air circulation

- Removal of smoke, heat, and grease

Poor ventilation leads to discomfort and reduced staff performance.

### 4. Lighting

- Bright lighting in preparation areas

- Natural lighting where possible

- Emergency lighting

Poor lighting increases risk of accidents and food contamination.

### 5. Plumbing and Drainage

- Adequate water supply

- Proper slope for drainage

- Grease traps

Poor drainage causes bad odors and hygiene problems.

### 6. Safety Installations

- Fire suppression systems

- Gas leak detectors

- Emergency exits

- First aid kits

FACTORS WHICH INFLUENCE KITCHEN DESIGN

1 Size and extent of the menu

Before a kitchen is planned and designed the management must know its goal and objectives in relation to the market strategy. In other wards what markets are you aiming at and what style of service or operation are you going to use? The menu will then determine the type of equipment you will require in order to produce the products that you know from the market research that the customer is going to buy as well as the target numbers that you intend to serve.

2 Service areas

The designer must know where service areas are located and how efficient use can be made of them.

3 Labor and skill level

What kind of people does the company intend to employ? This will have an effect on the technology and equipment to be installed.

Highly skilled chefs require:

- Specialized workstations

- Advanced equipment

Semi-skilled staff need:

- Simplified layout

- Clear workflow

The number of staff affects:

- Space allocation

- Locker rooms

- Rest areas

4 Amount of capital expenditure

Most design has to work with a detailed capital budget. This is because it is not always possible to design and then worry about costs afterwards.

Finance will very often determine the overall design and acceptability.

Because space is at a premium kitchen are generally smaller, equipment being designed to cater for the available space.

5 Use of prepared convenience foods

The fast food kitchen using prepared convenient food will influence the planning and equipping very differently from a’lacarte or cook-chill kitchen. This means certain factors will have to be determined such as; will sweets and pastries be made on the premises? will there be space for ladder? (butcher). Will fresh or frozen food or combination of both be used?

6 Type of equipment available

The type, amount and size of the equipment will depend on the type of the menu being provided.The equipment must be suitably sited.

7 Hygiene and food safety acts/1990/91/95

Design and construction of the kitchen must comply with the hygiene and the food safety act 1990/91/95.

The basic lay out and construction should enable adequate space to be provided in all food handling and associated areas as well as equipment and working practices.

## Available Space

The building structure influences Layout type ,Equipment arrangement, Storage capacity.Limited space may require vertical shelving and compact equipment.

## Budget and Financial Resources

Budget affects:

- Quality of materials

- Level of automation

- Kitchen technology

A limited budget may lead to basic equipment, while a higher budget allows modern combination ovens and energy-efficient systems.

## 6. Technology Level

Modern kitchens integrate:

- Digital temperature monitoring

- Automated dishwashers

- Combination ovens

- Inventory management systems

Advanced technology increases efficiency but requires skilled staff.

## 7. Health and Legal Regulations

Kitchen design must comply with:

- Public health laws

- Fire safety regulations

- Occupational health standards

In Uganda, local authority guidelines influence drainage, waste disposal, and hygiene design.

## 8. Utility Services Availability

Kitchen design depends on:

- Electricity capacity

- Gas supply

- Water reliability

- Drainage systems

Unreliable utilities may require backup generators or alternative energy sources.

## 9. Hygiene and Food Safety Standards

Proper design must ensure:

- Separation of raw and cooked food zones

- Adequate hand washing stations

- Pest-proof storage

- Waste segregation

This prevents foodborne diseases.

## 11. Future Expansion

Good planning considers:

- Increased production demand

- New menu introduction

- Technological upgrades

Flexibility reduces renovation costs.

## 12. Environmental Considerations

Modern kitchens focus on:

- Energy efficiency

- Water conservation

- Waste management

- Noise reduction

Sustainable design reduces operational costs and environmental impact.

# Overall Conclusion

Kitchen planning and designing is a strategic operational decision that directly influences:

- Food quality

- Staff productivity

- Hygiene standards

- Customer satisfaction

- Profitability

A well-designed kitchen ensures smooth workflow, safety, cost control, and long-term sustainability.

## Objectives of Kitchen Planning

- To ensure smooth workflow (from receiving to service).

- To minimize cross-contamination.

- To maximize space utilization.

- To reduce labor fatigue and unnecessary movement.

- To enhance supervision and control.

- To comply with health and safety regulations.

- To improve speed and quality of service.

## Principles of Kitchen Planning

### (a) Workflow Principle

The kitchen should follow a logical food flow:
Receiving → Storage → Preparation → Cooking → Service → Dishwashing

Food preoperational rooms should be planned to allow a work flow where by food is processed through the premises from the point of delivery to the point of sell or service with the minimum obstruction. The various processes should be separated as far as possible and food intended for sell should not cross paths with waste or refuse. This prevents backtracking and congestion.

Large operation should work on separate work flows, for reasons of efficiency and hygiene in terms of;

- Product. Raw materials to finished product.

- Personnel. How people move within the kitchen eg staff working in dirty areas (areas of contamination) should not enter areas of finished products/ where blast-chilling.

- Container/ equipment/ utensils. Where possible should be separated out into specific process area.

- Product flows

Each section should be divided into high risk and contaminated sections. High risk is that food which during the process is likely to be easily contaminated. While contaminated

- Access to ancillary areas.

A good receiving area needs to be designed for easy receipt to supply with in nearby storage facilities suitably sited for distribution of foods for preparation and production areas.

### (b) Work Triangle Principle

In smaller kitchens, the relationship between Storage areas, Preparation area, cooking areas should minimize unnecessary movement.

### (c)Hygiene and Food Safety

Hygiene is paramount so that kitchen equipment can be cleaned and stored at all times. Still room facilities may also be required.

Planning must ensure:

- Separation of raw and cooked foods

- Proper drainage

- Adequate ventilation

- Pest prevention systems

Humidity

The humid atmosphere creates side effect such as food deterioration, infestation risks, condensation walls and slippery floors. Anything higher than 60% humidity lowers productivity.

Provision for replacement of extracted area with fresh air is essential.

Temperature:

No higher than20-26 0C(68-79Of) is desirable for maximum working efficiency and comfort with 16-18 0c(61-64Of) in preparation areas’

Maintenance:

Planning and   equipping a kitchen is an expensive investment, therefore to avoid any action by the environmental health officer, efficient, regular clearing and maintenance is essential.

Optimum ventilation with fresh air intake

The working area must be suitably lit and ventilated with extractor funs to reduce heat,fumes,smoke and steam

This supports HACCP principles.

### (d) Cost Control

Good planning reduces:

- Energy consumption

- Wastage

- Equipment damage

- Labour costs

(e) Work place safety design features

- Non slip floor materials, fire alarm systems, fireextinguishers, electrical circuit breakers and Proper gas installationshould be put in to consideration to ensure safety of the staff

### (f) Space Utilization

The Office, Shop and Railway Premises Act of 1963, stipulates that 11.3m (4000ft) per person discounting height in excess of 3 m. Efficient use of vertical and horizontal space without overcrowding.

### (g) Flexibility

The kitchen should allow future expansion or technological upgrades.

(h) Equipment

The type, amount and size of equipment will depend on the type of menu being provided.Not only the equipment be suitably fitted but the working weight is very important so as to enable the equipment to be used without excess fatigue. Adequate refrigerators and freezer space

i)Light:

Maximum legal   level in preparation areas is 20 lumens per square ft with up to 38 lumens preferable in all areas.

The management must have easy access to the areas under control and have good visibility in the areas which have to be supervised.

TYPES OF THE LAYOUT OF COOKING PREMISES (AREAS)

# 1. ASSEMBLY LINE (STRAIGHT LINE) LAYOUT

Suitable for: Institutional catering, school kitchens, fast food operations
Main Advantage: Smooth forward workflow
Limitation: Limited flexibility

---------------------------------------------------------
| Receiving | Storage | Prep | Cooking | Service | Wash |
 ---------------------------------------------------------
                 → → → → → → (Food Flow Direction)

### Explanation:

- Equipment arranged in a straight sequence.

- Food moves in one direction.

- Minimizes cross-contamination.

- Ideal for high-volume standardized production.

2. U OR L shaped layouts;

Here arrangements create self-contained sections that discourage entry by non-authorized staff and promote efficient working with distances reduced between working centres.

When planning the layout of the cooking area, the need to allow sufficient space for access to equipment such as ovens should be borne in mind.

# U-SHAPED LAYOUT

Suitable for: Small to medium-sized kitchens
Main Advantage: Reduces movement
Limitation: Can be congested if space is small

____________________________
| Storage | Prep | Cooking  |
|                              |
| Wash     | Service |        |
|__________|_________|________|

### Explanation:

- Workstations arranged in a “U” form.

- Staff stand in the center.

- Easy access to all sections.

- Efficient for small teams.

# L-SHAPED LAYOUT

Suitable for: Limited space kitchens
Main Advantage: Efficient corner utilization
Limitation: Not ideal for very high production

______________________
| Storage | Prep       |
|                     |
|                     |
| Cooking | Service  |
|_________|___________|

### Explanation:

- Equipment arranged along two adjacent walls.

- Provides open space in the middle.

- Reduces traffic congestion.

# 3.ISLAND LAYOUT

In an island grouping, equipment is planned back to back in the Centre of the working area. This calls for a need to there be sufficient space to allow this, including adequate gang ways around the equipment and space to place other items along the walls.

Suitable for: Large hotels and high-class restaurants
Main Advantage: Central supervision
Limitation: Requires large space

_______________________________________
| Storage | Prep Area | Service        |
|                                        |
|          [ COOKING ISLAND ]           |
|                                        |
| Dishwashing | Cold Room | Dry Store  |
|_______________________________________|

### Explanation:

- Cooking equipment placed at the center.

- Preparation and storage areas around the perimeter.

- Promotes teamwork and supervision.

- Common in modern hotel kitchens.

# 4. ZONE-STYLE LAYOUT

Suitable for: Large hotel kitchens with specialization
Main Advantage: Specialization and efficiency
Limitation: Requires skilled staff and more space

_______________________________________________
| Butchery | Garde Manger | Pastry | Bakery    |
|-----------------------------------------------|
|               Hot Kitchen                    |
|-----------------------------------------------|
| Wash-up | Service Area | Storage | Receiving |
|_______________________________________________|

### Explanation:

- Kitchen divided into specialized production zones.

- Each section handles specific food types.

- Reduces interference between sections.

- Suitable for hotels with banqueting and multiple restaurants.

# 5. GALLEY (PARALLEL) LAYOUT

Suitable for: Narrow spaces (ships, small restaurants)
Main Advantage: Maximum space utilization
Limitation: Congestion during peak hours

___________________________________
| Storage | Prep | Cooking         |
|-----------------------------------|
| Wash    | Service | Refrigeration|
|___________________________________|

### Explanation:

- Two parallel counters.

- Narrow central walkway.

- Efficient for small teams.

6. SINGLE WALL SITTING

An alternative arrangement which involves sitting equipment along walls.This arrangement is possible where travel distances are reduced and normally occurs in small premises/areas